IMO MEETING REPORT

| DATE: 3rd July 2025 | COMMITTEE: MSC |
| --- | --- |
| ATTENDEES: Andy Williams | SUB GROUP: |

This was the 110th session of the Maritime Safety Committee (MSC) held from the 18th to 27th June 2025. I attended remotely on 20th June. ITEMS OF INTEREST TO THE LEISURE/SUPERYACHT INDUSTRY Revised recommendations for entering enclosed spaces – The Committee adopted the MSC resolution on amendments to the Revised recommendations for entering enclosed spaces aboard ships (resolution A.1050(27)). These recommendations support the SOLAS requirements for drills and the carriage of atmosphere testing equipment and apply to all ships, including yachts. The key changes from a yachting perspective are: - Competent person and responsible person training recommendations - Specific requirements on enclosed space entry training and drills from SOLAS regulation III/19.3.6. - A recommendation that the enclosed space contingency plan, based on the enclosed space register and risk assessment is reviewed after each drill - Both crew and shore-based personnel should fully understand the nature of the hazards that may be present - The validity of the enclosed space entry Permit To Work should never exceed 8 hours - Addition of specific levels of gas thresholds - Additional training requirements - A recommendation to avoid enclosed space entry outside of normal day time working hours. The text of the revised recommendations can be found at annex 5 of the report of the CCC Sub-Committee in attached document CCC 10/16. Development of a safety regulatory framework to support the reduction of GHG emissions from ships using new technologies and alternative fuels - The Committee reviewed and approved the report from the Working Group on GHG Safety, focusing on enhancing safety regulations for ships using alternative fuels and new technologies to reduce greenhouse gas emissions. The following key actions were taken: - Agreed to refer a document highlighting the need for an amendment to MSC.1/Circ.1621 (Interim Guidelines for the safety of ships using methyl/ethyl alcohol as fuel) to the CCC Sub-Committee for consideration. - Endorsed a list of recommendations to address regulatory gaps and barriers related to alternative fuels and technologies. - Agreed the instructions for how tasks are distributed to CCC, SSE, and SDC sub-committees, with flexibility in how they manage their workload as follows: - CCC assigned to review and develop safety measures for alcohol-based fuels and other alternatives. - SSE tasked with evaluating equipment-related safety for new fuel types. - SDC charged with updating design standards, including nuclear propulsion safety. - MEPC and PPR informed about safety concerns with Fatty-acid methyl ester (FAME). - ISO notified of missing standards for HTL fuel, Pyrolysis fuel, FT diesel, and Ammonia under ISO/TC 8. - The Committee acknowledged the need to revise the Code of Safety for Nuclear Merchant Ships to reflect modern technologies and safety standards. Full details of the actions taken and the status of this agenda item can be found in the attached report of the working group on the development of a safety regulatory framework to support the reduction of GHG emissions from ships using new technologies and alternative fuels in document MSC 110/WP.9. Cyber risk management and maritime security – The Committee agreed to develop a non-mandatory Cybersecurity Code which, once adopted, would be followed by an experience-building phase. A decision would then be made on whether the code should be mandatory. It was agreed that: - The code should be goal-based - The code should utilise a risk-management approach - The code should not be prescriptive in nature Proposals for a new output on the development of the non-mandatory code were invited for submission to MSC 111. The code should apply to ships and port facilities and should take account of the following: - Guidance in the development of the non-mandatory MASS Code - The work done by the FAL Committee regarding the Maritime Single Window (MSW) and the development of the comprehensive strategy on maritime digitalisation - Cybersecurity training requirements in the revision of the STCW Convention and Code Any mandatory cyber security requirements in the future would be mandated by a new chapter to the SOLAS Convention. For further details, please see the attached report of the working group on cybersecurity and maritime security in attached document MSC 110/WP.10. Maritime Autonomous Surface Ships (MASS) – The Committee continued work on the development of the non-mandatory Code to regulate Maritime Autonomous Surface Ships (MASS) to ensure these autonomous ships operate safely and in coexistence with conventional ships. The code has been finalised except for chapter 4, “Definitions” and chapter 15 “Human Element”. The draft text of the code can be found at annex 1 of the attached report of the working group in document MSC 110/WP.8. The Committee agreed the amended road map for developing a goal-based code as follows: Adoption of the non-mandatory code is expected to be at MSC 111 in May 2026. There will be an intersessional MASS working group (ISWG/MASS 4) to be held from 29th September to 3rd October 2025, the terms of reference of which can be found at annex 3 of the attached report of the working group ISM-related matters – The Committee decided to undertake a thorough update of the guidelines for implementing the ISM Code by Administrations and companies, specifically targeting resolution A.1188(33) on Guidelines for Administrations and MSC-MEPC.7/Circ.8 on Revised Guidelines for Companies' operational implementation of the ISM Code. Additionally, the Committee agreed to add an item titled "Comprehensive revision of the guidelines on the implementation of the ISM Code by Administrations and companies" to the III Sub-Committee's biennial agenda for 2026-2027 and the provisional agenda for III 12, with a completion target of 2028. The III Sub-Committee will lead this effort, collaborating with the HTW Sub-Committee as needed, and invited MEPC 84 to serve as a parent organ for this initiative. Draft report - The draft report of the Committee is contained in attached documents MSC.110/WP.1. Circulars approved at MSC 110 MSC.1/Circ.1428/Rev.1; Required pilot transfer arrangements for pilots and other personnel. MSC.1/Circ.[…]; Voluntary early implementation of the amendments to SOLAS regulation V/23. MSC.1/Circ.1266/Rev.1; Carriage of dangerous goods. MSC.1/Circ.1264/Rev.1; Recommendations on the safe use of pesticides in ships (fumigation of cargo holds). MSC.1/Circ.1358/Rev.1; Recommendations on the safe use of pesticides in ships. MSC.1/Circ.1395/Rev.7; Lists of solid bulk cargoes and fire-extinguishing system exemption/ineffectiveness. MSC.1/Circ.1601/Rev.2; Revised industry counter piracy guidance. MSC.1/Circ.[…]; Interim guidelines for emergency towing arrangements on ships other than tankers. MSC.1/Circ.1175/Rev.2; Revised guidance on shipboard towing and mooring equipment. MSC.1/Circ.1255/Rev.1; Revised guidelines for owners/operators on preparing emergency towing procedures. MSC.1/Circ.1502/Rev.1; Revised guidance on pressure testing of boundaries of cargo oil tanks. MSC.1/Circ.1331/Rev.1; Revised guidelines for construction, installation, maintenance and inspection of means of embarkation/disembarkation. MSC.1/Circ.[…]; Unified interpretation of SOLAS regulation II-1/12.6.2. MSC.1/Circ.[…]; Escape arrangements from the lower part of machinery spaces (SOLAS II-2/13.4.1 and 13.4.2). STCW.7/Circ.[25]; Generic interim guidelines on training for seafarers using alternative fuels and new technologies. MSC.1/Circ.1164/Rev.30; Promulgation of reports of independent evaluation confirming full and complete effect to STCW Convention provisions. MSC.1/Circ.797/Rev.42; List of competent persons maintained by the Secretary-General (STCW Code section A-I/7). MSC.1/Circ.1086/Rev.1; Revised Code of practice for atmospheric oil mist detectors. MSC.1/Circ.[…]; Unified interpretations of LSA Code paragraphs 6.1.1.3 and 6.1.2.2. MSC.1/Circ.[…]; Unified interpretations of SOLAS chapter II-2 and the 1994/2000 HSC Codes. MSC.1/Circ.[…]; Unified interpretation of the FSS Code. MSC.1/Circ.[…]; Unified interpretation of SOLAS regulation II-1/3-13.2.4. MSC.1/Circ.1628/Rev.4; Revised standardized life-saving appliance evaluation and test report forms (personal life-saving appliances). MSC.8/Circ.[3]; Rules of Procedure of the Maritime Safety Committee. MSC-MEPC.1/Circ.5/Rev.7; Draft amendments to organization/method of work of MSC and MEPC and subsidiary bodies. MSC.1/Circ.1587/Rev.1; Procedural aspects related to drafting amendments to IMO safety conventions (other than SOLAS 1974). PRINCIPAL ISSUES: The meeting agenda was as follows: - Adoption of the agenda - Decisions of other IMO bodies - Consideration and adoption of amendments to mandatory instruments - Goal-based new ship construction standards - Development of a goal-based instrument for Maritime Autonomous Surface Ships (MASS) - Development of a safety regulatory framework to support the reduction of GHG emissions from ships using new technologies and alternative fuels - Revision of the Guidelines on Maritime Cyber Risk Management (MSC-FAL.1/Circ.3/Rev.2) and identification of next steps to enhance maritime cybersecurity - Measures to enhance maritime security - Piracy and armed robbery against ships - Unsafe mixed migration by sea - Ship design and construction (Report of the eleventh session of the Sub-Committee) - Pollution Prevention and Response (Report of the twelfth session of the Sub -Committee) - Human element, training and watchkeeping (Report of the eleventh session of the Sub-Committee) - Ship systems and equipment (Report of the eleventh session of the Sub-Committee) - Navigation, communications and search and rescue (Urgent matters emanating from the eleventh session of the Sub-Committee) - Carriage of cargoes and containers (report of the tenth session of the Sub-Committee) - Application of the Committee’s method of work - Work programme - Election of Chair and Vice-Chair for 2026 - Any other business - Consideration of the report of the Committee on its 110th session Three working groups and one drafting group were established as follows: - Working Group on Maritime Autonomous Surface Ships (MASS). The terms of reference of the group were to: - Develop further the draft MASS Code, and to consider, in particular: - Chapters 1 to 5, 8, 14 and 15 of the draft MASS Code, based on annex 1 of document MSC 110/5, and taking into account the information provided in the body of the report. - Chapters 6, 9 to 11, 13 and 16 in part 2 of the draft MASS Code, based on the annex to document MSC 109/WP.8, including principles, application and necessary definitions, to confirm alignment and consistency with part 3. - Chapters 17bis, 19, 21 to 27 in part 3 of the draft MASS Code, based on the annex to document MSC 109/WP.8, taking into account the Generic guidelines for developing IMO goal based standards (MSC.1/Circ.1394/Rev.2) for Tier I and Tier II, and to ensure consistency and that all necessary IMO instruments are addressed, as appropriate. - If time permits, consider the potential gap in the draft MASS Code on delegation of the master's tasks and duties, taking into account paragraph 17 of document MSC 109/5. - Update the revised road map, based on annex 3 of document MSC 109/22/Add.1, if necessary. - Consider the intersessional working arrangements if necessary and prepare terms of reference for the intersessional working group and/or correspondence group, as appropriate. - Advise the Committee on how to keep documents MSC 110/5/3, on sole look-out, and MSC 110/5/20, on Experience-Building Phase (EBP) framework, in abeyance, with a view for future consideration. - Working Group on Development of a safety regulatory framework to support the reduction of GHG emissions from ships using new technologies and alternative fuels. The terms of reference of this group were to: - Taking into consideration annex 1 of document MSC 110/6; and the relevant comments contained in documents MSC 110/6/3, MSC 110/6/5, MSC 110/INF.19, MSC 110/INF.27, MSC 110/INF.28 and MSC 110/INF.30, update the list of recommendations to address existing barriers and gaps related to alternative fuels and new technologies. - based on document MSC 109/WP.9, progress on the development of recommendations to address the 14 additional gaps identified by MSC 109 in the areas of hydrothermal liquefaction (HTL) fuel, pyrolysis fuel, Fischer-Tropsch (FT) diesel, and swappable traction lithium-ion battery containers. - Based on the 32 recommendations provided in annex 1 of document MSC 110/6, and taking into account the criteria for the assignment of work to sub-committees (MSC 109/WP.9/Add.1), develop a list of priorities to be used as a guide to instruct the appropriate sub-committees to initiate work on specific identified alternative fuels or new technologies. - With regard to alternative fuels on gas carriers "one ship, one code" policy, consider options 1 and 3 (MSC 110/J/9) and determine important pros and cons to advise the Committee on the best way forward to decide on the application of this policy and report back to plenary by Monday, 23 June 2025. - Considering documents MSC 110/6 (specifically paragraphs 34 to 36 and annex 1); MSC 110/6/11; MSC 110/6/4; and MSC 110/18/16 (relevant sections pertaining to nuclear propulsion), prepare draft clear instructions for sub-committees to initiate the revision of SOLAS chapter VIII, in conjunction with the Nuclear Code (resolution A.491(XII)); with regard to safety issues only. - Discuss as necessary the work plan for Development of a safety regulatory framework to support the reduction of GHG emissions from ships using new technologies and alternative fuels to advise the Committees accordingly. - Consider the need to establish a correspondence group and, if deemed necessary, prepare draft terms of reference accordingly. 3. Working Group on Cyber Security and Maritime Security. The terms of reference of this group were: - Develop the approach on the next steps to enhance maritime cybersecurity, i.e. risk-based/goal-based/prescriptive. - Consider if cybersecurity requirements should be made mandatory or voluntary, and consider the IMO instrument that would be the most appropriate mechanism for implementing any new requirements. - Consider whether any interim measures are required, taking into account existing guidance to enhance maritime cybersecurity while associated requirements are under development. - Develop terms of reference for a correspondence group. - Consider and finalize the proposal in annex 1 to document MSC 110/8/1 on promoting the use of international maritime information-sharing centres to enhance maritime security and to assist in the fight against organized crime threats in the sector, based on document MSC 110/8/1, and advise the Committee on the type of instrument to be used (circular or resolution) for adoption by the Committee. 4. Drafting Group on Amendments to mandatory instruments. The terms of reference of this group were: Under agenda item 3 - Finalize the draft amendments to SOLAS chapters II-2 and V, and the draft associated MSC resolution, based on annex 2 of document MSC 110/WP.5. - Finalize the draft amendments to the 1994 and 2000 HSC Codes, and the draft associated MSC resolutions, based on annexes 3 and 4, respectively, of document MSC 110/WP.5. - Finalize the draft amendments to the IMSBC Code, and the draft associated MSC resolution, based on annex 6 of document MSC 110/WP.5, taking into account Circular Letter No.4925. - Finalize the draft performance standards for pilot transfer arrangements, and the draft associated MSC resolution, based on annex 7 of document MSC 110/WP.5, taking into account documents MSC 110/3/4, MSC 110/3/13 and MSC 110/3/14. - Finalize the draft amendments to the appendix (Form SPS) to the annex (Certificates) of the 2008 SPS Code, and the draft associated MSC resolution, based on annex 8 of document MSC 110/WP.5. - Finalize the draft amendments to the Code of Safety for Fishermen and Fishing Vessels, 2005, based on annex 9 of document MSC 110/WP.5. - Finalize, based on annexes 10 to 15 of document MSC 110/WP.5, the draft MSC circulars on: - Required pilot transfer arrangements for pilots and other personnel (MSC.1/Circ.1428/Rev.1) (MSC 110/WP.5, annex 10 and MSC 110/3/3). - Voluntary early implementation of the amendments to SOLAS regulation V/23 on pilot transfer arrangements, adopted by resolution MSC [...(110)] (MSC 110/WP.5, annex 11). - Carriage of dangerous goods, for dissemination as MSC.1/Circ.1266/Rev.1 (MSC 110/WP.5, annex 12). - Recommendations on the safe use of pesticides in ships applicable to the fumigation of cargo holds, for dissemination as MSC.1/Circ.1264/Rev.1 (MSC 110/WP.5, annex 13). - Recommendations on the safe use of pesticides in ships, for dissemination as MSC.1/Circ.1358/Rev.1 (MSC 110/WP.5, annex 14). - Lists of solid bulk cargoes for which a fixed gas fire-extinguishing system may be exempted or for which a fixed gas fire-extinguishing system is ineffective, for dissemination as MSC.1/Circ.1395/Rev.7 (MSC 110/WP.5, annex 15). for consideration by the Committee, with a view to adoption or approval, as appropriate, on Thursday, 26 June 2025, so that the expanded Committee may consider and adopt/approve them on that day with duly translated texts in French and Spanish. - Further progress draft amendments to the IGC Code, and the draft associated MSC resolution, in line with the decisions made earlier, based on annex 5 to document MSC 110/WP.5, taking into account editorial proposals in document MSC 110/3/2. - Prepare draft amendments to SOLAS chapter II-1 in relation to the definitions of "low-flashpoint fuel" and "IGF Code", and regulations 55 and 56 and consequential draft amendments to the IGF Code, taking into account document MSC 110/6/6, and MSC 110/J/12, with a view to approval at this session. - Consider the assessment of capacity-building implications related to the draft amendments to SOLAS regulation V/23 and associated instruments contained in the annex to document MSC 109/13/4, and advise the Committee with a view to endorsement of the assessment, as appropriate. - Assess the implications for capacity-building and technical cooperation and assistance of the amendments to the following mandatory instruments submitted for adoption at this session, against the revised procedures and criteria for identification of capacity-building implications set out in annex 2 of the Committees' method of work (MSC-MEPC.1/Circ.5/Rev.6): - Draft amendments to SOLAS chapter II-2 (MSC 110/WP.5, annex 2). - Draft amendments to the 1994 and 2000 HSC Codes (MSC 110/WP.5, annexes 3 and 4, respectively). - Draft amendments to the IMSBC Code (MSC 110/WP.5, annex 6). - If applicable, provide a description of the potential capacity-building implications of new or amended instruments along with recommendations for a course of action, for consideration and action by the Technical Cooperation Committee, as appropriate. Under agenda item 14 - Prepare the application provisions in the draft amendments to the LSA Code in the annex to document MSC 110/14/1 and annex 1 of document SSE 11/20, taking into account documents MSC 110/14/2 (option 2 on defining "installed on or after") and the proposals in paragraphs 12 and 13 of document MSC 110/14/3, and its annex, with a view to approval.

ATTACHMENTS/SUPPORTING DOCUMENTATION: